How to setup BSM 9.2x
with 
Service Health Analyzer (SHA) Data Collector for Performance Agent (PA)

Table of Contents

Overview	2

Test Environment	3

Integrate SiteScope with BSM	4

Setup Operations Agent integration with BSM	5

Verification : does OA (embedded PA and/or Coda) collect data?	10

Verification : does BSM recognize the Operations Agent instance?	13

Verification : can BSM graph OA / PA metrics via PMi?	14

Install BSM SHA Data Collector for Performance Agent	15

JMX for SHA Data Collector	19

Enable / configure SHA to collect metrics from PA	20

What metrics does SHA DC for PA collect by default	24

TQL “Hosts with HP Operations Agents – Discovery” doesn’t work	25

Verify that SHA collects PA metrics	27

Running SHA DC for PA without installing it	30

# Overview

This document gives an overview how to setup the BSM SHA Data Collector for PA,
including the steps required to integrate SiteScope with BSM, the Operations Agent with SiteScope,
and all other required setup steps plus some troubleshooting information.

# Test Environment

- BSM 9.21 is installed on a Windows 2008 system, 
   system name is vm17094

- SHA is licensed and enabled via Server Deployment


- SHA database has been defined

- SiteScope 11.20 with Operations Agent is installed on a Windows 2008 system,
   system name is vm25174
   SiteScope 11.20 carries Operations Agent 11.02.011
   Operations Agent 11.x also includes the Performance Agent

# Integrate SiteScope with BSM

# Setup Operations Agent integration with BSM

Clicking on “Connect” will run opcactivate, which does a lot of magic behind the scenes.

Please make sure to follow the comments in the output:

as one needs do execute (on the BSM server)
- ovcm –listpending –l
  this will return the certificate request from the SiteScope server
  take a note of the request_id and then execute
- ovcm –grant <request_id from above>

Next is to click on “Install Policies”

If all this worked as expected, click on “Analyze” and make sure you see this line:


If the line says something like “Message Agent is buffering locally” or alike,
it means the agent cannot talk to the BSM server, for example the certificate request has not been granted yet.

To check whether or not the event integration to BSM / OMi works,
use these two tests:
“Sent Test Message” uses the OA’s tool opcmsg to send a message


This should result in the following popup

and this entry in BSM -> Applications -> Operations Management -> Event Perspective

Similar test:
“Sent Test Event” writes a line into the integration log file which is then analyzed by the policies deployed before

Note:
This type of automated integration displays all OA commands it uses,
so all this can be done manually as well, or commands can be copied and used if needed, for example
  bbcutil.exe -ping <bsmserver>
  opcagt.bat  -status
  ovcert -check
  ovpolicy -list

# Verification : does OA (embedded PA and/or Coda) collect data?

Are the processes up and running?

C:\Users\Administrator>perfstat
******************************************************
** perfstat for VM25174  Thu Jan 31 14:00:17 2013
** Microsoft Windows   [ Version 6.1.7600 ]
******************************************************
List of Performance Tool Processes:
===================================
  pid:  1100    dsisrv             (DSI service)
  pid:  1588    midaemon           (measurement interface)
  pid:  1544    misrv              (measurement interface service)
  pid:  2012    perfalarm          (Alarm generator)
  pid:  1980    perfalarmsrv       (Alarm generator service)
  pid:  2784    scopent            (data collector)
  pid:  2720    scopesrv           (collector service)
  pid:   352    perfd              (Real Time Metric Access Daemon)
  pid:   880    perfdsrv           (Real Time Metric Access Service)
  pid:  2420    ttd                (transaction tracking)
  pid:  2356    ttsrv              (transaction tracking service)
  pid:  1384    coda               (CODA performance component)

        Configured Datasources[1]
        SCOPE

pid:  5896    rtmd               (Real Time Measurement daemon)
  pid:  5548    ovbbccb            (BBC5 communication broker)
  pid:   560    ovcd               (OV control component)

All critical MeasureWare processes are executing

MeasureWare Extended Collection status
  pid:  1756    Mwecsrv            (extended collection service)

All critical MeasureWare Extended Collection processes are executing
=============  (end of perfstat -p output) ===========

Data can be found in the collector logfiles. These are located at
%ovdatadir%datafiles

C:\Users\Administrator>ovcodautil –showds
NumDataSources = 2
SCOPE
CODA

C:\Users\Administrator>ovcodautil –dumpcoda
=== 01/31/13 2:00:00 PM
..
GBL_CPU_TOTAL_UTIL                : 5.24
..

# Verification : does BSM recognize the Operations Agent instance?

- Go to BSM -> Administration -> RTSM -> IT Universe Manager
   select the view “System Software Monitoring”:

or
- Go to BSM -> Applications -> Service Health -> 360° View
   select view  “System Software Monitoring”:


Note:
the view “Hosts with HP Operations Agents” ONLY shows system which have been setup via a HP OM system.
As in this example we integrated the Operations Agent with BSM directly, it doesn’t show up in this very view:

# Verification : can BSM graph OA / PA metrics via PMi?

If all works as expected, one now can graph OA / PA metrics via the BSM Performance Perspective:

# Install BSM SHA Data Collector for Performance Agent

Finally SHA DC for PA needs to be installed

Note:
if you plan to install the SHA DC for PA on the same system running the Operations Agent, you need to FIRST install the Operations Agent (here SiteScope with the Operations Agent),
otherwise the Operations Agent installation will fail due to version issues with multiple Ov packages.

Note:
SHA PA DataCollector is a strip-down version of BSM, and thus cannot be installed on a system where BSM has been installed already.

To install SHA PA Data Collector:
execute setup.exe

If you get this message

IIS or Apache might be active on the system, using port 80.
Other then with full BSM, SHA DC post-install configuration wizard does NOT allow the modification of the port, so better make sure that the port is free during installation.

The remaining steps for the installation now are just to tell SHA DC what database to connect to (that’s the BSM Management database from the BSM installation running SHA) and the JMX password.

After the installation completed successfully, SHA DC needs to be started.
This is done the same way BSM is started:

The progress and status of the startup can easily be checked by monitoring the logfile
<BSM>\log\supervisor\nannyStatus.all
…
2013-01-31 12:35:12,266  analytics_dc             STOPPED
2013-01-31 12:35:12,267  analytics_dc             LAUNCHED
2013-01-31 12:35:16,650  analytics_dc             STARTING
2013-01-31 12:35:16,651  analytics_dc             STARTED
2013-01-31 12:35:16,775  All services started in 4s

Now SHA DC for PA is ready to be used and can be configured.

# JMX for SHA Data Collector

SHA DC offers a JMX Console via port 29931, as one can learn from the logfile
<BSM>\log\analytics_dc_boot.log
2013-01-31 12:35:22,852 - HttpAdaptor version 3.0.2 started on port 29931

http://<system with SHA DC for PA installed>:29931

# Enable / configure SHA to collect metrics from PA

To enable SHA to collect metrics from PA, a Business Service needs to be created and CIs assigned to it.
First create a Business Service via BSM -> Admin -> Service Level Management -> Services Manager  and assign the system running OA to it:



Then this Business Service is added to the SHA CI Selection:

Important note:
The Data Collector collects data from the agents it finds via the view/TQL “Hosts with HP Operations Agents”. The agent’s node CI must be part of the CIs in the CI selection.
If the agent has not been registered via an OM server but to BSM directly (like in this example), the Infrastructure Settings need to be changed to use the TQL 
“Hosts with HP Operations Agents – Discovery”.
This requires that Discovery reported the agent:

BSM -> Administration -> Platform -> Infrastructure Settings
select 
 Foundations : Analytics Data Collector
change
 PA Agents view name from
   “Hosts with HP Operations Agents”
 to
   “Hosts with HP Operations Agents – Discovery” (without quotes)
and save the change.

After this change, run the refreshPAAgents JMX:
for this access
 http://<system with SHA DC for PA installed>:29931
  Topaz:service=PA data collector
    invoke java.lang.String refreshPAAgents

If everything is configured properly, one will see this message:
1 agents for customer id 1, IPs are: [vm25174.deu.hp.com]

If not, one will see:
0 agents for customer id 1, IPs are: []

Now check the PA Agent Status
for this access
 http://<system with SHA DC for PA installed>:29931
  Topaz:service=PA data collector
    invoke java.lang.String agentsStatus
    customerId : 1

Agent{ciId=a4bd48c386fe4ff54211c63b673e784d, agentAddress='vm25174.deu.hp.com', average samples per 5 minutes in the last hour=[SCOPE: 28 ], status=AgentStatus{lastBadConnectionTime=, lastGoodConnectionTime=Thu Mar 07 19:03:14 CET 2013, lastGoodDataAccessTime=Thu Mar 07 19:03:14 CET 2013, lastBadDataAccessTime=, dataAccessDurationTime=64, createConnectionDurationTime=61, numberOfFailedConnectionRetries=0, numberOfFailedDataRetries=0, numberOfSuccessConnectionRetries=2, numberOfSuccessDataRetries=2, poolingInterval=AgentPoolingInterval{timeUnit='MINUTES', interval=5}, lastTimeFrameTo=1362679394565, state=DATA_POSTED}, connection=AgentConnection{codaObjects={

{-1 SCOPE SCOPE C:\ProgramData\HP\HP BTO Software\\datafiles\logglob={Object name : NETIF  11=AgentCodaObject{metrics=[BYNETIF_IN_PACKET_RATE, BYNETIF_OUT_PACKET_RATE], instances={Instance: ID = 0Number of strobes: 0Number of Keys: 1Instance Key: BYNETIF_NAME 14 DATA:class java.lang.String=a4bd48c386fe4ff54211c63b673e784d, Instance: ID = 1Number of strobes: 0Number of Keys: 1Instance Key: BYNETIF_NAME 14 DATA:class java.lang.String=a4bd48c386fe4ff54211c63b673e784d, Instance: ID = 2Number of strobes: 0Number of Keys: 1Instance Key: BYNETIF_NAME 14 DATA:class java.lang.String=a4bd48c386fe4ff54211c63b673e784d, Instance: ID = 3Number of strobes: 0Number of Keys: 1Instance Key: BYNETIF_NAME 14 DATA:class java.lang.String=a4bd48c386fe4ff54211c63b673e784d}}

, Object name : GLOBAL  1=AgentCodaObject{metrics=[GBL_SYSCALL_RATE, GBL_ACTIVE_CPU, GBL_CPU_TOTAL_UTIL, GBL_CPU_TOTAL_TIME, GBL_CPU_SYS_MODE_UTIL, GBL_CPU_SYS_MODE_TIME, GBL_CPU_USER_MODE_UTIL, GBL_CPU_USER_MODE_TIME, GBL_CPU_IDLE_UTIL, GBL_CPU_IDLE_TIME, GBL_DISK_PHYS_IO_RATE, GBL_DISK_UTIL_PEAK, GBL_DISK_LOGL_READ_RATE, GBL_DISK_PHYS_READ_RATE, GBL_DISK_PHYS_WRITE_RATE, GBL_FS_SPACE_UTIL_PEAK, GBL_DISK_REQUEST_QUEUE, GBL_MEM_UTIL, GBL_SWAP_SPACE_UTIL, GBL_MEM_PAGEOUT_RATE, GBL_NET_OUTQUEUE, GBL_NET_IN_PACKET_RATE, GBL_NET_OUT_PACKET_RATE, GBL_NET_PACKET_RATE, GBL_ACTIVE_PROC, GBL_RUN_QUEUE, GBL_LOADAVG, GBL_NET_UTIL_PEAK], instances={Instance: ID = 0 Number of strobes: 0 Number of Keys: 0=a4bd48c386fe4ff54211c63b673e784d}}

}}, codaInfo= **** CODA INFO ACCESS 
System Type + HostName + GMTOFFSET + STROBEINT + BUILDVER + BUILDATE + BBCVER 
4  vm25174  -60  8  11.02.009  00/00/00 11.03.031

, connectionInfo= null vm25174.deu.hp.com vm25174.deu.hp.com /Hewlett-Packard/OpenView/Coda/ false null null null 0 60}}
]

# What metrics does SHA DC for PA collect by default

SHA DC of PA collects a subset of the Performance Agent GLOBAL metrics, and predefined content for various SPIS.
This is controlled via Analytics Metadata, which is located at the DPS at
<BSM>\conf\analytics\metadata\default\PA.xml
    ..
    <metric-type name="PA_Global_Metrics" mapped-to="pa_metric_t">
                    ..
                    <match value="GBL_NET_IN_PACKET_RATE"/>
                    <match value="GBL_NET_OUT_PACKET_RATE"/>
                    ..
                    <match value="GBL_ACTIVE_PROC"/>
                    <match value="GBL_SYSCALL_RATE"/>
                    ..


   <metric-type name="PA_SPI_Metrics" mapped-to="pa_metric_t">
                    ..
                    <!-- webLogic -->
                    <match value="B005_JVMMEMUTILPCT"/> 
                    <match value="B012_EXQUEWAITCNT"/>
                    ..
                    <!-- webSphere -->
                    <match value="I005_JVMMEMUTILPCT"/>
                    <match value="I013_THRDPOOLPCTMAX"/>
                    ..
                    <!-- mssql -->
                    ..
                    <!-- oracle -->
                    ..
                    <!-- exchange -->
                    ..
                    <!-- domain controller [active directory] -->
                    ..
                    <!-- Virtualization -->
                    ..

# TQL “Hosts with HP Operations Agents – Discovery” doesn’t work

It has been observed at least once that the TQL “Hosts with HP Operations Agents – Discovery” doesn’t work.
After changing the Infrastructure Settings and refreshing the agent as described, the logfile
<HPBSM>\log\odb\odb\cmdb.api.audit.detailed.log
shows
query: ExecutableQueryImpl{queryName='Hosts with HP Operations Agents – Discovery', queryDefinition=null, multiRefDef=null, parentQueryName='null', queryRestrictions={}, wholePatternPropertiesCarrierSupport={Properties carrier: {default: behavior: PROPERTIES_AND_QUALIFIERS named: 
..
Error processing request:
Pattern [Hosts with HP Operations Agents – Discovery] doesn't exist
com.mercury.topaz.cmdb.shared.tql.exception.TqlException: [ErrorCode [55034] TQL doesn't exist]
Pattern [Hosts with HP Operations Agents – Discovery] doesn't exist

While the TQL works fine and can be used in RTSM Admin

it doesn’t work when being used externally.
The workaround here is to save the TQL under a different name (for example “SG_PA_Hosts”) and then use this TQL in the Infrastructure Settings:

# Verify that SHA collects PA metrics

Note:
for more details please see 
KM00374312 - SHA - information around metrics and baselines

http://<DPS>:29926/mbean?objectname=Topaz%3Aservice%3DAnalytics+CIs
 java.lang.String refresh
with
 customerId : 1
and click Invoke:

customer id 1: 39 CIs; 
1 CIs of type nt in samples [nnm_metric_t, ss_t, pa_metric_t, diag_metric_t]
{a4bd48c386fe4ff54211c63b673e784d=vm25174}

17 CIs of type business_transaction in samples [trans_t, rum_trans_t]
..

1 CIs of type hp_operations_agent in samples [rum_application_stats_t, rum_tcp_application_stats_t, ss_t, rum_trans_t, pa_metric_t, diag_metric_t]
{f1b486e3dc312e879ec5d908de7e511c=Operations-agent on vm25174.deu.hp.com}

18 CIs of type sitescope_monitor in samples [ss_t]
..

1 CIs of type interface in samples [pa_metric_t]
{995deaa5291c86eeccc8ad0fefc87809=005056B94C2B}
..

 pa_metric_t is the data type for the Performance Agent metrics.

http://<DPS>:29924/mbean?objectname=Topaz%3Aservice%3DBaseline+Services
 java.lang.String showDomainNames
enter
 customerId : 1
and click Invoke

Domain names for 1:
SCOM
RUM
BPM
SiS
NNM
PA
DIAG

Again, PA is the domain name we are interested in.

http://<DPS>:29924/mbean?objectname=Topaz%3Aservice%3DBaseline+Services
 java.lang.String showMetricTypeNames
enter
  customerId : 1
  domainName : PA
and click Invoke

Metric type names for customerID 1 domainName PA:

PA_SPI_Metrics

PA_Global_Metrics

So we can see that SHA is aware of the PA metrics, metric type "PA_SPI_Metrics" and "PA_Global_Metrics", using the next step we can see what PA metrics are collected:

http://<DPS>:29924/mbean?objectname=Topaz%3Aservice%3DBaseline+Services
  java.lang.String showMetricsOfType
   customerID     : 1
   domainName     : PA
   metricTypeName : PA_Global_Metrics 

Metrics of customerID 1 domainName PA metricTypeName PA_Global_Metrics:

bulk/metric: 3/1022 type: 1/PA/PA_Global_Metrics Dimensions: [a4bd48c386fe4ff54211c63b673e784d, GBL_ACTIVE_CPU, SCOPE, GLOBAL, localhost [0]]
..
bulk/metric: 3/1005 type: 1/PA/PA_Global_Metrics Dimensions: [a4bd48c386fe4ff54211c63b673e784d, GBL_CPU_TOTAL_UTIL, SCOPE, GLOBAL, localhost [0]]
..
bulk/metric: 3/1028 type: 1/PA/PA_Global_Metrics Dimensions: [a4bd48c386fe4ff54211c63b673e784d, GBL_SYSCALL_RATE, SCOPE, GLOBAL, localhost [0]]

Note:
most of the data above can be retrieved more easily with running the agentStatus JMX

To verify that SHA collected and stored metric data in the SHA database for metric GBL_CPU_TOTAL_UTIL, execute the following query against the SHA database:

SELECT top 5 *

FROM AA_RAW

where metric_id = 1005 and bulk_id = 3

order by sampletime desc

Sampletime  Metric_Id  BULK_ID  VAL       …

1362730500  1005       3        3.070000

1362730200  1005       3        3.540000

1362729900  1005       3        3.150000

1362729600  1005       3        3.100000

1362729300  1005       3        3.420000

Sampletime is returned as Unix Epoch time (seconds since 1/1/1970) and can be converted by (for example) http://www.epochconverter.com/

1362730500 = Fri, 08 Mar 2013 08:15:00 GMT

Check whether a baseline exists for GBL_CPU_TOTAL_UTIL http://<DPS>:29924/mbean?objectname=Topaz%3Aservice%3DBaseline+Services  

java.lang.String extractBaselineResult

metricID : 1005

bulkID    : 3

Baseline:

Learning Time Frame: Sat Feb 02 00:00:00 CET 2013 - Mon Mar 04 00:00:00 CET 2013

Trend (change per day:) 0.0

Confidence Level: 96.948118

Buckets:

0: mean=2.5643, std-dev=0.219

# Running SHA DC for PA without installing it

For a test, SHA DC for PA can also be enabled on the Gateway server, so there is no need for a separate installation. All what needs to be done is to enable it via

http://<<GW>>:11021/invoke?operation=enableService&objectname=Foundations%3Atype%3DNannyManager&value0=analytics_dc&type0=java.lang.String 

When doing this one needs to make sure that the total amount of agents is small, to support a large number of agents you need the separate machine.